FREQUENCY MODULATION

2.1. Generation and Analysis of Frequency Modulated (FM) Signals Using MATLAB

AIM :  To generate and study  Generation and Analysis of Frequency Modulated (FM) Signals Using MATLAB .


SOFTWARE REQUIRED : MATLAB 

Objective 1: To generate a Frequency Modulated (FM) signal using MATLAB by varying the frequency of a high-frequency carrier in accordance with the amplitude of the message signal and to observe the resulting waveform in the time domain.

CODE : 
clc;

clear;

close all;

Am=1;

Ac=1;

fm=5;

fc=50;

beta=5;

kp=5;

fs=1000;

t=0:1/fs:1;

m = Am*cos(2*pi*fm*t);              	% Message signal

c = Ac*cos(2*pi*fc*t);              	% Carrier signal

fm_sig = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); % FM

subplot(4,1,1), plot(t,m), title('Message Signal')

subplot(4,1,2), plot(t,c), title('Carrier Signal')

subplot(4,1,3), plot(t,fm_sig), title('FM Signal')

OUTPUT :

Objective 2: To analyze the frequency spectrum of the generated FM signal using FFT and identify the carrier component, sideband frequencies, and the bandwidth of the FM signal based on Carson's rule

Code : 
% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signals

clc;

clear;

close all;

%% Input Parameters

Am = 2;              % Message signal amplitude

fm = 500;            % Message frequency (Hz)

Ac = 1;              % Carrier amplitude

fc = 10000;          % Carrier frequency (Hz)

kf = 1000;           % Frequency sensitivity (Hz/V)

fs = 100000;         % Sampling frequency (Hz)

T = 0.02;            % Simulation duration (s)

t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation Calculation

delta_f = kf * Am;

%% Modulation Index Calculation

beta = delta_f/fm;

%% FM Signal Generation

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Plot Message Signal

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% Plot FM Signal

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1.5);

title('Frequency Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% Spectrum Analysis

N = length(fm_signal);

FM_fft = fftshift(abs(fft(fm_signal))/N);

f = (-N/2:N/2-1)*(fs/N);

figure;

plot(f,FM_fft,'LineWidth',1.5);

title('FM Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

xlim([fc-10000 fc+10000]);

%% Display Results

fprintf('\nFM SIGNAL PARAMETERS\n');

fprintf('---------------------------\n');

fprintf('Carrier Frequency      = %.0f Hz\n',fc);

fprintf('Message Frequency      = %.0f Hz\n',fm);

fprintf('Message Amplitude      = %.2f V\n',Am);

fprintf('Frequency Sensitivity  = %.0f Hz/V\n',kf);

fprintf('\nCALCULATED VALUES\n');

fprintf('---------------------------\n');

fprintf('Frequency Deviation (Δf) = %.0f Hz\n',delta_f);

fprintf('Modulation Index (β)     = %.2f\n',beta);

%% Carson's Rule Bandwidth

BW = 2*(delta_f + fm);

fprintf('Bandwidth (Carson Rule) = %.0f Hz\n',BW);

Out put :

RESULTS : This program is successfully executed using the matlab .

2.2. CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION

AIM :  To generate and study  Frequency Deviation and Modulation Index of FM Signals Using MATLAB  .


SOFTWARE REQUIRED : MATLAB 

Objectives:

To generate a Frequency Modulated (FM) signal in MATLAB using specified message and carrier signal parameters, and to determine the frequency deviation and modulation index of the FM signal while analyzing the relationship between frequency deviation, message frequency, and modulation index.

CODE : 
% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signal

clc;

clear;

close all;

%% Message Signal Parameters

Am = 2;      	% Message amplitude (V)

fm = 500;    	% Message frequency (Hz)

%% Carrier Signal Parameters

Ac = 1;      	% Carrier amplitude (V)

fc = 10000;  	% Carrier frequency (Hz)

%% FM Parameters

kf = 250;    	% Frequency sensitivity (Hz/V)

fs = 100000; 	% Sampling frequency (Hz)

T = 0.01;    	% Simulation duration (s)

%% Time Vector

t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation

delta_f = kf*Am;

%% Modulation Index

beta = delta_f/fm;

%% FM Signal

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Display Parameters

fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc);

fprintf('Message Frequency (fm) = %.0f Hz\n',fm);

fprintf('Message Amplitude (Am) = %.0f V\n',Am);

fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf);

fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f);

fprintf('Modulation Index (Beta) = %.2f\n',beta);

%% Verification

beta_verify = delta_f/fm;

fprintf('Verified Modulation Index = %.2f\n',beta_verify);

%% Verification Check

tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance

fprintf('\nVerification Successful\n');

else

fprintf('\nVerification Failed\n');

end

%% Plotting

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

grid on;

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1);

grid on;

title('FM Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

OUTPUT :

RESULTS : This program is successfully executed using the matlab .